Smart Temperature & Humidity Monitor

Complete Project Guide — Arduino Uno + DHT11 + I2C LCD ¦ Beginner Level ¦ Cost: ~Rs.300-500

## Project Overview

Build a real-time temperature and humidity monitoring station using a DHT11 sensor and a 16x2 LCD display. The system reads values every 2 seconds, displays them on screen, and triggers a serial alert when temperature or humidity exceeds a set threshold. This is one of the most practical beginner IoT projects — the skills you learn here (I2C, sensor drivers, thresholds) appear in almost every real embedded product.

| Difficulty | Time | Cost |
| --- | --- | --- |
| Beginner | 1-2 weekends | Rs.300-500 |

## What You Will Learn

- Reading sensor data using a digital protocol (DHT one-wire)

- Communicating with an LCD display over I2C (just 2 wires)

- Displaying formatted text and numbers on an LCD

- Implementing threshold-based alert logic in C

- Using the Serial monitor for debugging and logging

- Library installation and management in Arduino IDE

# Section 1 — Components & Tools

## Components List

| # | Component | Purpose | Qty | Cost (Rs.) |
| --- | --- | --- | --- | --- |
| 1 | Arduino Uno R3 | Main microcontroller board | 1 | Rs.500-800 |
| 2 | DHT11 Sensor | Temperature and humidity sensing | 1 | Rs.80-150 |
| 3 | 16x2 I2C LCD Module | Display for readings | 1 | Rs.150-220 |
| 4 | Breadboard (half size) | Prototyping connections | 1 | Rs.60-100 |
| 5 | Jumper wires (M-M) | Circuit connections | 10-15 | Rs.30-60 |
| 6 | 10k Ohm resistor | Pull-up for DHT11 data line | 1 | Rs.2-5 |
| 7 | USB cable (A to B) | Power and programming | 1 | Already owned |

NOTE: Buy the I2C version of the 16x2 LCD (it has a small PCF8574 adapter board on the back). This uses only 2 wires (SDA and SCL) instead of 6. If you accidentally buy the non-I2C version, you will need 6 digital pins and a different library.

## Software Required

| Software | Library | How to Install |
| --- | --- | --- |
| Arduino IDE 2.x | DHT sensor library (Adafruit) | Tools > Manage Libraries > search "DHT sensor library" |
| Arduino IDE 2.x | Adafruit Unified Sensor | Tools > Manage Libraries > search "Adafruit Unified Sensor" |
| Arduino IDE 2.x | LiquidCrystal_I2C | Tools > Manage Libraries > search "LiquidCrystal I2C" by Frank de Brabander |

TIP: Install all three libraries before writing any code. Missing libraries are the #1 cause of "compilation failed" errors for beginners.

# Section 2 — Circuit Wiring

Wire all components before uploading any code. It is much easier to debug wiring problems without code running. Follow the tables below precisely.

## DHT11 Sensor Wiring

| DHT11 Pin | Arduino Pin | Wire Color | Notes |
| --- | --- | --- | --- |
| Pin 1 (VCC) | 5V | Red | Power supply |
| Pin 2 (DATA) | Digital Pin 2 | Yellow / Green | 10k pull-up resistor between DATA and VCC |
| Pin 3 (NC) | Not connected | — | Leave floating |
| Pin 4 (GND) | GND | Black | Ground |

## I2C LCD Wiring

| LCD Pin | Arduino Pin | Wire Color | Notes |
| --- | --- | --- | --- |
| GND | GND | Black | Ground |
| VCC | 5V | Red | Power supply |
| SDA | Analog Pin A4 | Blue | I2C data line |
| SCL | Analog Pin A5 | Yellow | I2C clock line |

NOTE: The I2C LCD address is usually 0x27 or 0x3F. If your LCD shows nothing after uploading code, try changing the address in the code. The I2C Scanner sketch (see Section 5) will tell you the exact address.

## Pull-up Resistor Explained

The DHT11 data pin uses a "single-wire" protocol. Without a pull-up resistor, the data line floats and gives garbage readings. Place a 10k ohm resistor between the DATA pin and the 5V rail on the breadboard. This is a one-time hardware setup, not something done in software.

## Wiring Checklist

- DHT11 VCC connected to Arduino 5V

- DHT11 GND connected to Arduino GND

- DHT11 DATA connected to Arduino Digital Pin 2

- 10k resistor bridging DHT11 DATA and 5V rail

- LCD GND connected to Arduino GND

- LCD VCC connected to Arduino 5V

- LCD SDA connected to Arduino A4

- LCD SCL connected to Arduino A5

- Arduino connected to PC via USB

# Section 3 — Complete Arduino Code

Copy the code below exactly into a new Arduino IDE sketch. Read the inline comments carefully — each section is explained so you understand what every line does.

## Full Project Code

Arduino Code

// ============================================================

// Smart Temperature & Humidity Monitor

// Hardware: Arduino Uno, DHT11, 16x2 I2C LCD

// ============================================================

// --- Libraries ---

#include <DHT.h> // DHT sensor driver

#include <LiquidCrystal_I2C.h> // I2C LCD driver

// --- Pin & Sensor Configuration ---

#define DHT_PIN 2 // DHT11 data wire -> Arduino pin 2

#define DHT_TYPE DHT11 // We are using DHT11 (not DHT22)

// --- Threshold Values (adjust to your needs) ---

#define TEMP_HIGH 35 // Alert if temp exceeds 35 deg C

#define TEMP_LOW 10 // Alert if temp drops below 10 deg C

#define HUM_HIGH 80 // Alert if humidity exceeds 80%

#define HUM_LOW 20 // Alert if humidity drops below 20%

// --- I2C LCD Setup ---

// Arguments: (I2C address, columns, rows)

// Try 0x3F if 0x27 does not work on your module

LiquidCrystal_I2C lcd(0x27, 16, 2);

// --- DHT Sensor Object ---

DHT dht(DHT_PIN, DHT_TYPE);

// --- Custom degree symbol for LCD ---

byte degreeChar[8] = {

B00110, B01001, B01001, B00110,

B00000, B00000, B00000, B00000

};

// ============================================================

void setup() {

Serial.begin(9600); // Open serial port at 9600 baud

Serial.println("Smart Temp & Humidity Monitor Starting...");

dht.begin(); // Initialize DHT11 sensor

lcd.init(); // Initialize LCD

lcd.backlight(); // Turn on backlight

lcd.createChar(0, degreeChar); // Register custom degree symbol

// Startup splash screen

lcd.setCursor(0, 0);

lcd.print(" Temp Monitor ");

lcd.setCursor(0, 1);

lcd.print(" Initializing ");

delay(2000);

lcd.clear();

}

// ============================================================

void loop() {

// Wait 2 seconds between readings

// DHT11 needs at least 1 second between samples

delay(2000);

// Read temperature and humidity

float humidity = dht.readHumidity();

float temperature = dht.readTemperature(); // Celsius by default

// Check if readings are valid (NaN = Not a Number = read failed)

if (isnan(humidity) ¦¦ isnan(temperature)) {

Serial.println("ERROR: Failed to read from DHT11!");

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Sensor Error!");

lcd.setCursor(0, 1);

lcd.print("Check wiring.");

return; // Skip rest of loop, try again next cycle

}

// ---- Display on LCD ----

lcd.clear();

// Row 0: Temperature

lcd.setCursor(0, 0);

lcd.print("Temp: ");

lcd.print(temperature, 1); // 1 decimal place

lcd.write(0); // Custom degree character

lcd.print("C");

// Row 1: Humidity

lcd.setCursor(0, 1);

lcd.print("Hum: ");

lcd.print(humidity, 1);

lcd.print(" %");

// ---- Log to Serial Monitor ----

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.print(" C ¦ Humidity: ");

Serial.print(humidity);

Serial.println(" %");

// ---- Threshold Alerts ----

if (temperature > TEMP_HIGH) {

Serial.println("!! ALERT: Temperature TOO HIGH !!");

lcd.setCursor(14, 0);

lcd.print("!!");

}

if (temperature < TEMP_LOW) {

Serial.println("!! ALERT: Temperature TOO LOW !!");

lcd.setCursor(14, 0);

lcd.print("Lo");

}

if (humidity > HUM_HIGH) {

Serial.println("!! ALERT: Humidity TOO HIGH !!");

lcd.setCursor(14, 1);

lcd.print("!!");

}

if (humidity < HUM_LOW) {

Serial.println("!! ALERT: Humidity TOO LOW !!");

lcd.setCursor(14, 1);

lcd.print("Lo");

}

}

// ============================================================

// End of sketch

## Code Walkthrough

| Code Section | What It Does |
| --- | --- |
| #include | Loads the DHT and LCD libraries you installed. Without these, none of the sensor or display functions exist. |
| #define DHT_PIN 2 | Gives the number 2 a readable name. If you wire DHT to a different pin, change only this one line. |
| #define TEMP_HIGH 35 | Sets your alert thresholds. Change these numbers to suit your environment. |
| LiquidCrystal_I2C lcd(0x27,...) | Creates the LCD object. The 0x27 is the I2C address. Change to 0x3F if display shows nothing. |
| dht.begin() / lcd.init() | Initialises both devices. Must run before any reading or display commands. |
| dht.readHumidity() | Returns a float (decimal number) from 20.0 to 90.0. Takes about 250ms to respond. |
| isnan() | Checks if reading failed. DHT11 returns NaN (not a number) when wiring is wrong or timing fails. |
| lcd.setCursor(col, row) | Moves cursor to column (0-15) and row (0-1). Always call before lcd.print(). |
| lcd.write(0) | Prints the custom degree symbol you defined in degreeChar array. |
| Serial.println() | Sends text to Serial Monitor. Open with Ctrl+Shift+M in Arduino IDE. |

# Section 4 — Upload & Test

## Step-by-Step Upload Process

- Open Arduino IDE 2.x on your computer.

- Go to File > New Sketch. Delete the default empty code.

- Copy the entire code from Section 3 and paste it into the editor.

- Connect your Arduino Uno to the PC via USB cable.

- Select board: Tools > Board > Arduino AVR Boards > Arduino Uno.

- Select port: Tools > Port > COM3 (Windows) or /dev/ttyUSB0 (Linux/Mac). Pick the one that appears when the Arduino is plugged in.

- Click the Upload button (right arrow icon) or press Ctrl+U.

- Wait for "Done uploading" message at the bottom of IDE.

- Open Serial Monitor: Tools > Serial Monitor (or Ctrl+Shift+M). Set baud rate to 9600.

## Expected Results

| What You Should See | Where |
| --- | --- |
| "Temp Monitor / Initializing" splash screen for 2 seconds | On the LCD immediately after power-on |
| "Temp: 28.5 C Hum: 65.0 %" (your actual room values) | On the LCD, updating every 2 seconds |
| "Temperature: 28.5 C ¦ Humidity: 65.0 %" log line | In the Serial Monitor, every 2 seconds |
| "!! ALERT: Temperature TOO HIGH !!" message | In Serial Monitor if temp goes above 35C |
| "!!" appearing at right side of LCD row | On LCD when alert is triggered |

TIP: Hold your thumb on the DHT11 sensor for 10 seconds to raise the temperature and humidity readings. This is a quick way to test that your threshold alerts are working correctly without waiting for environmental changes.

# Section 5 — Troubleshooting

| Problem | Most Likely Cause | Fix |
| --- | --- | --- |
| LCD shows nothing / blank | Wrong I2C address or no backlight | Change 0x27 to 0x3F in code. Turn contrast pot on LCD back until text appears. |
| LCD shows black squares | Contrast too high | Adjust the blue potentiometer screw on the I2C adapter board with a small screwdriver. |
| "Sensor Error!" on LCD | DHT11 wiring issue | Check pull-up resistor is in place. Verify DHT pin 2 matches #define DHT_PIN 2 in code. |
| Serial monitor shows garbage | Wrong baud rate | Set Serial Monitor baud rate dropdown to 9600. |
| Compilation error "no such file" | Library not installed | Install all three libraries listed in Section 1. Restart Arduino IDE after installing. |
| Port not showing in Tools menu | Driver not installed / cable issue | Install CH340 driver for clone boards. Try a different USB cable (some are charge-only). |
| Temperature reads 0.0 always | Library version mismatch | Use DHT sensor library by Adafruit specifically. Delete any other DHT libraries. |

## I2C Address Scanner

If your LCD is not responding, upload this separate sketch to find its exact I2C address. Open Serial Monitor at 9600 baud after uploading.

Arduino Code

#include <Wire.h>

void setup() {

Wire.begin();

Serial.begin(9600);

Serial.println("Scanning for I2C devices...");

for (byte addr = 8; addr < 127; addr++) {

Wire.beginTransmission(addr);

if (Wire.endTransmission() == 0) {

Serial.print("Found device at address: 0x");

Serial.println(addr, HEX);

}

}

Serial.println("Scan complete.");

}

void loop() {}

# Section 6 — Upgrades & Next Steps

Once the basic project is working, try these upgrades in order. Each one teaches a new concept that builds on the previous.

| Level | Upgrade | What It Teaches | New Component | Extra Cost |
| --- | --- | --- | --- | --- |
| 1 | Add a buzzer alert | PWM output, tone() function | Passive buzzer | Rs.15-30 |
| 2 | Add a second row cycling display | LCD multi-screen, millis() timing | None | Free |
| 3 | Add heat index calculation | Floating point math, derived values | None | Free |
| 4 | Log data to SD card | SPI protocol, file systems, CSV format | SD card module | Rs.150-250 |
| 5 | Send data over WiFi to ThingSpeak | ESP-AT commands, HTTP requests, IoT cloud | ESP8266 module | Rs.200-350 |
| 6 | Replace DHT11 with DHT22 | Higher accuracy sensors, library reuse | DHT22 sensor | Rs.150-250 |

## Concepts You Have Mastered

- I2C communication protocol — the same protocol used in countless professional sensors

- Single-wire sensor protocol (DHT) — understand pull-up resistors and signal integrity

- Threshold-based logic — the foundation of all alarm and control systems

- Serial debugging — the primary tool professionals use to diagnose embedded firmware

- Library-based development — reading documentation and trusting proven code

- Floating point display formatting — printing decimals cleanly on constrained displays

You have built a real embedded IoT device from scratch. The skills used here — I2C, sensors, thresholds, serial logging — appear in professional embedded products every day.